MECHATRONICS TRAINING SET TENDER SPECIFICATIONS

- The training set should be designed to provide showing students how to deal with various types of sensors and how to set an algorithm with a PLC.

- There should be various types of samples that are detected by sensors. The samples on the motor driven belt can be catagorized and eliminated via the program in the PLC and the touch panel.

- RF-ID tags should be integrated into the samples. The choices of the categorization should vary between sensors and RF-ID tags.

- RF-ID reader should detect at least 4 different samples.

- The choice of the categorization need to be made via the touch screen.

- The samples ejected from the sample parts magazine should be tranferred to the categorization part via pneumatic rotary actuator.

- The system should include the following equipments.

- RF-ID unit

- Ultrasonic sensor for height measurement

- Fibre optical sensor which has response time of 50µs and can detect transperent objects

- Color detection sensor

- Inductive sensor

- Magnetic sensor

- PLC, Siemens S7-224XP

- Sample program

- Touch screen, colored, 3,5”

- Motor driven belt

- Pressure measurement

- Vacuum measurement

- Vacuum unit

- Pneumatic cylinder

- Pneumatic rotary actuator

- Electropneumatic valves

- The following training objectives need to be covered.

- Basic programming and understanding of an automation process flow chart

- Learning the use of industrial programming and communicating with a PLC

- Scada software programming to visualise a real mini industrial process.

- Pressure control via a pressure switch and a mechanical manometer

- Vacuum measurement via a digital vacuum sensor with LCD screen

- Arrange the related solenoid valves according to the present scenario

- Setup and piping of preesurised air systems

- Understanding the sensor characteristics

- The structure of the training set should be made from anodised aluminium profile. All aluminium profiles should be suitable for M8 standart nut and should not need any special nuts.