PNEUMATIC-ELECTROPNEUMATIC TRAINING SET TENDER SPECIFICATIONS

- The training set should be in an expandable form for pneumatic, electro-pneumatic, proportional pneumatic and PLC applications.

- All equipments and pipes need to be in the form of easily mountable with pneumatic fittings.

- The equipments can be attached onto the aluminium panel via simple connection element.

- The dimension of the valves should be 1/8”.

- Operating pressure of the system should be 6 bars.

- Workstation need to be suitable to use double-sided, with adding an extra aluminium panel.

- Dimensions of the workstation should be at least 700mm x 1.200mm x 1900mm.

- The structure of the trolley should be made of welded steelprofiles and electrostatic painted.

- Dimensions of the aluminium panel need to be 700mm x 1.000mm.

- “T” slotted anodised aluminium panels should be suitable for M8 standart nut and should not need any special nuts.

- Colons of the workstation should be 90x90 anodized aluminium sigma profile and the slots should be suitable for M8 nuts.

- Top panels of the control modules should be made up of nonconductive material and mid-box  should be non-metal.

- Dimensions of the triad drawer unit should be at least 500mm x 500mm x 600mm.

- Connection blocks of the valves and cylinders should be made up of aluminium.

- Power supply of the compressor motor should be 220 VAC, and the motor power should be at least 0,37 kW.

- There need to be pressure emergency valve, manometer and oil indicator on the compressor.

- All electrical connection terminals of the system should be 4mm isolated banana jacks.

- The following training objectives need to be covered.

- Fundamentals of pneumatic & electropneumatic circuits

- Air preperation

- Single and double acting cylinders

- Air piloted directional control valves

- Mechanically actuated directional control valves

- Logic pneumatic valves

- Speed control valves

- Pressure adjusting regulators and pressure sequence valves

- Timer valves

- Understanding of vacuum, generated from pressurised air, with venturi principle

- Signal eliminations

- Solenoid operated valves

- Proportional technology with pressure and flow

- Circuit configuration with relays and logic electric moduls

- Learning the use of industrial programming and communicating with a PLC

- Learning the use of touchscreen to visualise and control circuits

- Arrange the related solenoid valves according to sample scenarios

- Setup and piping of preesurised air systems, pipes can be installed easily by quick connections.

- Understanding the sensor characteristics

- There should be at least the following list to cover the basic pneumatic applications.

| No | Description | Quantity |
| --- | --- | --- |
| 1. | 3/2 way valve push button - NC | 2 |
| 2. | 3/2 way valve push button - NO | 1 |
| 3. | 3/2 way valve selector switch- NO | 1 |
| 4. | 5/2 way valve selector switch | 1 |
| 5. | 3/2 way roller lever valve -NC | 2 |
| 6. | 3/2 way single acting pneumatic valve NC | 1 |
| 7. | 3/2 way pneumatic proximity valve -NC | 2 |
| 8. | 5/2 way single acting pneumatic valve | 1 |
| 9. | 5/2 way double acting pneumatic valve | 3 |
| 10. | OR valve | 1 |
| 11. | AND valve | 2 |
| 12. | 3/2 time delay valve - NO / NC | 1 |
| 13. | Quick exhaust valve | 1 |
| 14. | One way flow control valve | 2 |
| 15. | 3/2 pressure sequence valve | 1 |
| 16. | Filter regulator unit | 1 |
| 17. | Pressure regulator | 1 |
| 18. | Manometer | 2 |
| 19. | Single acting cylinder (diameter 20mm, stroke 50 mm) | 1 |
| 20. | Double acting cylinder (diameter 20mm, stroke 50 mm) | 1 |
| 21. | Air distribution block | 1 |
| 22. | Polyurethane hose | 2 |
| 23. | T branch, quick push-in fittings | 6 |
| 24. | Hose cutter | 1 |
| 25. | Pneumatic-electropneumatic book | 1 |
| 26. | Pneumatic-electropneumatic exercise book | 1 |

- There should be at least the following list to cover the basic electro-pneumatic applications.

| No | Description | Quantity |
| --- | --- | --- |
| 1. | One way flow control valve | 4 |
| 2. | Filter regulator unit | 1 |
| 3. | Single acting cylinder with magnet (diameter 20mm, stroke 50 mm) | 1 |
| 4. | Double acting cylinder with magnet (diameter 20mm, stroke 100 mm) | 2 |
| 5. | Air distribution block | 1 |
| 6. | Polyurethane hose | 1 |
| 7. | Aluminium "T" slotted plate | 1 |
| 8. | Triad drawer | 1 |
| 9. | 3/2 way single acting solenoid valve NC | 1 |
| 10. | 5/2 way single acting solenoid valve | 1 |
| 11. | 5/2 way double acting solenoid valve | 2 |
| 12. | Digital pressure transducer with LCD | 1 |
| 13. | Proximity sensor for cylinder use | 2 |
| 14. | Optical sensor | 1 |
| 15. | Electrical limit switch both side actuated | 2 |
| 16. | Cable set | 1 |
| 17. | Power supply | 1 |
| 18. | Relay modul | 1 |
| 19. | Push button modul | 1 |

- The following list of equipments are supplementary to the basic pneumatic equipments.

| No | Description | Quantity |
| --- | --- | --- |
| 1. | 3/2 way valve push button - NC | 2 |
| 2. | 3/2 way emergency stop valve-NO | 1 |
| 3. | 3/2 way valve mech. actuated- NO | 1 |
| 4. | 3/2 wayroller lever valve single side actuated -NC | 1 |
| 5. | 3/2 way single acting pneumatic valve NC | 4 |
| 6. | 5/2 way double acting pneumatic valve | 2 |
| 7. | OR valve | 2 |
| 8. | AND valve | 2 |
| 9. | 3/2 time delay valve - NO / NC | 1 |
| 10. | One way flow control valve | 2 |
| 11. | Non return valve with pilot open | 2 |
| 12. | Double acting cylinder (diameter 20mm, stroke 50 mm) | 2 |
| 13. | Pneumatic counter | 1 |
| 14. | Polyurethane hose | 2 |
| 15. | T branch, quick push-in fittings | 2 |

- The following list of equipments are supplementary to the basic electro-pneumatic equipments.

| No | Description | Quantity |
| --- | --- | --- |
| 1. | Non return valve with pilot open | 2 |
| 2. | 5/2 way single acting solenoid valve | 2 |
| 3. | 5/2 way double acting solenoid valve | 2 |
| 4. | Pneumatic electric converter | 1 |
| 5. | Inductive sensor | 1 |
| 6. | Capasitive sensor | 1 |
| 7. | Relay modul | 1 |
| 8. | Push button modul | 1 |
| 9. | Counter and timer modul | 1 |