HYDRAULIC  ELECTROHYDRAULIC TRAINING SET TENDER SPECIFICATIONS

- The training set should be in an expandable form for hydraulic, electro-hydraulic, proportional hydraulic and PLC applications.

- All equipments and pipes need to be in the form of easily mountable with hydraulic quick couplings.

- The equipments can be attached onto the aluminium panel via simple connection element.

- The valves should be NG4 and the place of origin shoıld be Europe.

- Operating pressure of the system should be 60 bars and teh maximum pressure should be 100 bars.

- Workstation need to be suitable to use double-sided, with adding an extra aluminium panel.

- Dimensions of the workstation should be at least 700mm x 1.200mm x 1900mm.

- The structure of the trolley should be made of welded steelprofiles and electrostatic painted.

- Dimensions of the aluminium panel need to be 700mm x 1.000mm.

- “T” slotted anodised aluminium panels should be suitable for M8 standart nut and should not need any special nuts.

- Colons of the workstation should be 90x90 anodized aluminium sigma profile and the slots should be suitable for M8 nuts.

- Top panels of the control modules should be made up of nonconductive material and mid-box  should be non-metal.

- Dimensions of the triad drawer unit should be at least 500mm x 500mm x 600mm.

- Connection blocks of the valves and cylinders should be made up of aluminium.

- The codes of the equipments should be written on the blocks by dot matrix printer  via depressing.

- The symblos of the equipments, that do not have symbols on its, should be drawn by printer.

- Aluminium blocks need to be anodised against the crossing out.

- Power supply of the hydraulic power unit should be 220 VAC, and the motor power should be at least 0,75 kW.

- There need to be pressure emergency valve, manometer and oil indicator on the power unit.

- There need to be a gate valve to deplate the oil and a cover to fill in the oil.

- There need to be a temperature indicator to observe the oil temperature.

- All electrical connection terminals of the system should be 4mm isolated banana jacks.

- The following training objectives need to be covered.

- Fundamentals of hydraulic & electrohydraulic circuits

- Power unit setup, oil filter, tank, electric motor, pressure relief valve adjustment

- Cylinders and hydromotors

- Mechanically actuated directional control valves

- Speed control valves

- Pressure adjusting regulators and pressure sequence valves

- Gas pressurised accumulator use

- Flow dividers

- Non return valves

- Solenoid operated valves

- Proportional valves

- Circuit configuration with relays and logic electric moduls

- Learning the use of industrial programming and communicating with a PLC

- Learning the use of touchscreen to visualise and control circuits

- Arrange the related solenoid valves according to sample scenarios

- Setup and piping of preesurised oil systems, pipes can be installed easily by quick connections.

- Understanding the sensor characteristics

- There should be at least the following list to cover the basic hydraulic applications.

| No | Description | Quantity |
| --- | --- | --- |
| 1. | 4/2 way manual valve | 1 |
| 2. | 4/3 way manual valve mid position open | 1 |
| 3. | Pressure sequence valve | 1 |
| 4. | 3 way pressure reducing valve | 1 |
| 5. | Piloted pressure relief valve | 1 |
| 6. | Two way flow control valve | 1 |
| 7. | One way flow control valve | 1 |
| 8. | Non return valve | 1 |
| 9. | Non return valve with pilot open | 1 |
| 10. | Diaphragm accumulator with safety block | 1 |
| 11. | Hydromotor bidirectional | 1 |
| 12. | Double acting cylinder (diameter 20mm, stroke 200 mm) | 1 |
| 13. | Flow control valve pressure compansated | 1 |
| 14. | Distributor block, P,T | 1 |
| 15. | Manometer | 2 |
| 16. | Shut-off valve | 1 |
| 17. | T fitting witk quick couplings | 6 |
| 18. | Hose with quick coupling 500mm | 8 |
| 19. | HHHose with quick coupling 1000mm | 6 |
| 20. | Hose with quick coupling 1500mm | 4 |
| 21. | Hydraulic power unit | 1 |
| 22. | Pressure relief part | 1 |
| 23. | Hose holder | 1 |
| 24. | Hydraulic electrohydraulic book | 1 |
| 25. | Hydraulic electrohydraulic exercise book | 1 |

- There should be at least the following list to cover the basic electro-hydraulic applications.

| No | Description | Quantity |
| --- | --- | --- |
| 1. | Pressure relief valve | 1 |
| 2. | One way flow control valve | 1 |
| 3. | Non return valve | 1 |
| 4. | Non return valve with pilot open | 1 |
| 5. | Hydromotor bidirectional | 1 |
| 6. | Double acting cylinder (diameter 20mm, stroke 200 mm) | 1 |
| 7. | Manometer | 2 |
| 8. | Pressure relief part | 1 |
| 9. | 4/2 way solenoid valve | 1 |
| 10. | 4/3 way solenoid valve mid position closed | 1 |
| 11. | Electrical limit switch both side actuated | 4 |
| 12. | Cable set | 1 |
| 13. | Power supply | 1 |
| 14. | Relay modul | 2 |
| 15. | Push button modul | 1 |

- The following list of equipments are supplementary to the basic hydraulic equipments.

| No | Description | Quantity |
| --- | --- | --- |
| 1. | 4/3 way manual valve mid position closed | 1 |
| 2. | 4/3 way manual valve "y" center | 1 |
| 3. | Counter balance valve | 1 |
| 4. | Two way flow control valve | 1 |
| 5. | Flow divider | 1 |
| 6. | T fitting witk quick couplings | 6 |
| 7. | Hose with quick coupling 500mm | 8 |
| 8. | HHHose with quick coupling 1000mm | 6 |
| 9. | Hose with quick coupling 1500mm | 4 |

- The following list of equipments are supplementary to the basic electro-hydraulic equipments.

| No | Description | Quantity |
| --- | --- | --- |
| 1. | 4/2 way solenoid valve | 1 |
| 2. | Pressure switch | 1 |
| 3. | Inductive sensor | 1 |
| 4. | Capasitive sensor | 1 |
| 5. | Optical sensor | 1 |
| 6. | Cable set | 1 |
| 7. | Relay modul | 1 |
| 8. | Counter and timer modul | 1 |