SERVOMOTOR TRAINING SET TENDER SPECIFICATIONS

- The training set need to be designed to be able to upgraded for applications with PLC controlled servomotor, PLC-Pneumatic axis servomotor.

- Transport system should consist of railed car and belt drive.

- There need to be an emergency stop button to stop the system in  an unexpected situation. It should be located apart form the control panel.

- "T" slotted anodised aluminium panel channels should be suitable for M8 standart nut and should not need any special nut.

- The left and right  limits of the transport system (belt drive and railed car) should be sensed by inductive sensors. These need to be fixed into the alluminium slots via simple connection elements without any screwing.

- Technical data of servo motor;

- 400 W,

- 3000 rpm

- 1.274 Nm

- Technical data of servo driver;

- Single phase supply

- Operating modes; speed, torque, position, position/speed, speed/torque, position/torque

  - Speed/torque:  ±10V  for analog input, 0-3000rpm for digital input

  - Position: External (open collector or line driver) or internal position (16 positions assigned by digital inputs)

- 6 optional digital inputs, 3 optional digital outputs

- Speed torque limiting

- Windows based software

- RS232 / RS485 communication

- Absolute  / incremental position control

- Origin search and position instruction functions

- Internal breaking unit

- Notch filtered

- Protection functions and alarm messages

- Monitoring all inputs and outputs

- 2500 ppr encoder dissolubility

- Dimensions of the working platform need to be at least 500mm x 1.000mm x 750mm.

- The power supply of the system should be 220 VAC, 50/60 Hz.

- There need to be a control panel and should include the followings;

- Servo operation (ON / OFF)

- Alarm reset

- Speed/Torque choice

- Right/ Left movement direction choice

- +24VDC supply

- Speed and torque tuning potantiometers

- The system should consist of;

- Servomotor and servo drive

- Belt drive

- Inductive sensors

- Emergency stop

- Manual control Simulator

- The following training objectives need to be covered

- Servomotor and servo drive characteristics

- Learning the parameter adjusting of servo drive

- Position Torque and speed control with servomotor

- Servomotor encoders and signal reading

- Setup of the manual control unit with a servo drive, sensors and travelled lineer distance calculation from servomotor encoder with a display

- Understanding the sensor characteristics

- Training set should be supplied with, servo driver-PC communication cable and user manual.